C# Form Application

Contents

1.	Connection Form:	1

2.	Enter Data Form:	4

3.	Mixed Palletizing Form:	6

# Connection Form:

In the Connection form we have:

This table contains information of robot’s controller:

- IP Address –  Address of Controller

- ID – You will see the ID when the controller is not virtual

- Availability – Whether you can connect to the controller or not

- Virtual – True if the controller is virtual and vice versa

- Controller nam- Name of the controller

- Status – Whether connected or not

You must choose the controller that you want to connect before clicking Connect to Controller button.

In TCP we have the coordinate of end – effector if the 	controller is connected :

We have 5 button:

Connect to Controller button: Connect to the controller.

Disconnect to Controller button: Disconnect to the controller.

Start RAPID Program button: Run the Rapid and excecute the code..

Stop RAPID Program button: Stop the RAPID when you want to stop.

Motor On/Off button: Turn on/off the motor of real robot. If you want robot operate with RAPID, turn on the motor before Starting RAPID.

When you connected successful the color will be changed to green:

# Enter Data Form:

In the Enter data form we must fill out the input of he box:

Box has 6 atributes: Name, Width, Height, Depth, Weight, Number of Box.

After you fill all the information in this panel, clicking Submit button.

All the parameters of the box will be displayed in the table below:

If you want to delete box you must choose that box in the table and click Delete Items

Then click Create Boxes button (The station must be opened and connected before clicking Create Boxes button)

Boxes in Robotstudio will be created. See the picture below:

# Mixed Palletizing Form:

After Create box, clicking Solve. Wait for a while. The result will be dispalyed in the table.

There are 4 options in the table:

Each option has:

- Volume in excess: Uused Volume

- Fitted box: Total box that fit the pallet

- MSE Of COG: Error of the Center of Gravity

Then click Send Tarfet button: All target of the box  that has been paacked on the pallet will be sent to robotstudio.

In RAPID code the  tgPos will contains the target.